WinField Release 2025 – signed with digital certificate

Overview of the new features:

q	All executable (*.exe, *.dll) files of the software are digitally signed by certificate

q	The number of simultaneous calculation processes has been increased to 64

q	Extraction of maximum values also possible in zoom mode

q	Automatic expansion of incomplete ground profiles (DGM)

General Improvements:

- All executable (*.exe and *.dll) files of the software are digitally signed with a 3-year certificate from 'GlobalSign', a company licensed by 'Microsoft'. This guarantees firstly that the software originates from a software publisher and secondly it protects the software against changes after publication. This means that there can no longer be any complaints about security from the Windows OS or virus scanners.

Cartographical Data:

- Defective AGR ground profiles can now be subsequently edited to rectify errors. AGR ground profiles that are incomplete (bottom left image) can now be modified via the Option:

They are corrected with regard to impermissible (0) values and expanded to the edges (Fig. bottom right). The effect is that when this option is used, the 'missing ground values' are fitted to the real ground profile:

- It is checked whether (0) values are available at all (Fig. left), so that the dialog only appears if this is the case. The 'missing ground values' are extrapolated in order to approximately produce the real DGM (Fig. right):

- When importing several ground profiles at the same time, this works for *.agr, *.pro, *.xyz and *.txt format, as shown above. When importing only one ground profile, the option is only available for *.xyz and *.txt format. To apply the option to a single *.agr profile, it must first be loaded and saved as *.xyz format.

- General note: Calculations without DGM with tower positions at Z=0 are always approximately valid. With DGM, the accuracy only improves if the DGM is also correct and completely covers the course of the route. However, as the DGM is often incorrect, it should always be checked manually. This is because a field calculation with height Z=1m is carried out with DGM at 1m above the ground, i.e. if the ground model is too low compared to reality, the field values are also too low - or vice versa.

New Calculation Functions:

- The number of simultaneous program processes for execution has been increased to 64.

Improved User Interface:

- The handling of maximum values has been improved. In addition to the maximum value of the entire calculation (Fig. above), the local maximum value can now also be saved when zooming (Fig. below):

This allows the maximum to be successively marked for all mast fields:

This function has also been implemented for xz,yz plane views, including inclined planes.

Forschungsgesellschaft für Energie und Umwelttechnologie - FGEU mbH

Yorckstraße 60, D - 10965 Berlin, Telefon 030 / 786 9799, Telefax 030 / 786 6389